| Subsystem/ Item: | Requirements: | Demonstration Setup: | % Grade | % Earned |
| --- | --- | --- | --- | --- |
| Communication | - The subsystem must maintain a wireless connection between Forest Buddy and the base station at 200-300 ft.. - The subsystem must provide GNSS positional accuracy within a 10 cm radius around Forest Buddy. - The subsystem must allow remote control of Forest Buddy’s forward, backward, left, and right movements from 200-300 ft way. - The subsystem must allow Forest Buddy to respond to the movement commands in less than 2 seconds at 200-300 ft away. | - Move Forest Buddy 300 ft from the base station and verify wireless communication through successfully received messages. Perform 5 trials and write a report..(5%) - Record GNSS positions at a fixed location before and after receiving RTK corrections. Compare results to evaluate positional accuracy within 10 cm. Perform 5 trials and write a report.(5%) - At 300 ft, demonstrate forward, backward, left, and right movements using a remote control. Perform in a live demo.(5%) - Forest Buddy will be placed 300 ft from the remote controller. Command response time will be measured using a timer or video to verify movement begins within 2 seconds. A live demo will be performed.(5%) | 20% |  |
| Navigation/Localization | - The subsystem must measure the rover’s orientation within ±5 degrees of its true orientation. - The subsystem must use LiDAR to detect obstacles. When an obstacle is detected at 5 ft, the rover must begin slowing and stop within 1 second, leaving 1-3 ft between the rover and obstacle. - The subsystem must use a navigation algorithm to guide Forest Buddy through saved waypoints in the correct order, reaching within 2 ft of each waypoint without requiring the operator to always lead the rover. | - Forest Buddy will be placed on a level surface and its IMU orientation recorded. The rover will then be tilted forward, backward, left, and right. Each measured angle will be compared to the level surface to verify ± 5 degree accuracy. Five trials must meet all requirements.(6.67%) - An obstacle will be placed in the rover’s path. Starting more than 5 feet away, the rover will approach at a constant speed. Video will verify that slowing begins at 5 feet, stopping occurs within 1 second, and the final distance is 1-3 ft. Five trials must meet all requirements.(6.67%) - A predefined waypoint route will be loaded into Forest Buddy. The rover must navigate without operator guidance, reaching each waypoint in order and within 2 ft of its target position. Recorded positions will verfy accuracy during alive demo.(6.67%) | 20% |  |

| Locomotion | - The subsystem must be able to climb up a 20% graded slope. - The subsystem must be able to travel over obstacles less than 5” tall. - The subsystem must be able to stop the rover from full speed(est. 3.9 mph) to a full stop within 1 second. | - Forest Buddy will be placed at the bottom of a wedge at an angle of approximately 12 degrees and attempt to drive up and down the incline.(6.67%) - Several obstacles smaller than 5” will be fixed directly in front of Forest Buddy’s wheels, which will then attempt to drive over them.(6.67%) - Forest Buddy will be driven forward at maximum speed, then the rover will be commanded to stop. The rover must stop within 1 second of the stop command being issued.(6.67%) | 20% |  |
| --- | --- | --- | --- | --- |
| Demo | All three high-risk subsystems are ready to demo at the time scheduled. Each requirement and corresponding demonstration setups written above have been attempted . [40 %] |  | 40% |  |
| Total: |  |  | 100% |  |

NOTE: All team members must be present. If a team member is not present for this demonstration. Their score shall be 0% while the other team members will receive the grade allocated in this rubric. This is acknowledged by the team and all team members upon submission.